[image: image1.emf]   

 

 

　　　　　  (W)－８　
REVISED MAY,2018
　　　　　　　　　　　　　　　LICENSED INLAND SEA PILOT         　　
　　　　　　　　　　　 　　 PILOT　INFORMATION　CARD 
　　　　       Bridge　Resource　Management　Exchange　With　Pilot (s) 
To the Master of   　　　　　　　　        　 　     Pilot:  　　　　　　　　　　       　 
For　safe navigation of your vessel you are kindly requested to: 

‐ confirm the pilotage plan on this card,
‐ keep a sharp lookout,　　　　　　　　　　　　　　　　　　　  
‐ keep watching VHF on channel 16, and                       
‐ put a question whenever you acknowledge a discrepancy.      
    ‐ fix position proper interval.
[image: image2.png]


[image: image3.png]


Details of Passage Plan and relevant information
         　　　　　　　　　　　　　　　　　　
[image: image4.emf]A vessel underway or at anchor should display the followings. 

Day                             Night

Vessel of 200m      Two black                    Flashing  

or more in L.O.A.:cylinders green light

A vessel underway or at anchor should display the followings. 

Day                             Night

Vessel of 200m      Two black                    Flashing  

or more in L.O.A.:cylinders green light


[image: image5.png]about 150m off






   　　　　　　　　　　　　　　　　　　　 　　　 　 
Other Information
                                                                                  
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　
Safety Measures for berthing
① All tugs should be made fast to the ship before arriving harbor limit or at anchorage.
② Approaching speed at above 1000m from the berth should be adjust to about 2~3kts.
③ The ship should be made parallel and stop at about 150m off the jetty.
④ Final berthing speed should be kept less than 10cm/sec.
Berthing Plan of Iwakuni C-2 Terminal                  





Date:                        
                                         　            　                                 　　　
                        Master                                     Inland Sea Pilot
CHECK  LIST  UNDER  PILOTAGE

DATE           :
SHIP’S  NAME  :

Voy.No　　　　 
	1. CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT
	

	MAIN ENGINE
	

	SHIP’S DRAFT　　　　F：　　　　　 (M：　　　　　)　 A：              
	

	WHISTLE (IF AIR SUPPLY & LOCATION)
	

	THRUSTER IF INSTALL
	

	STEERING GEARS UNDER OPERATION (TWO UNITS)
	

	GYRO COMPASS (INCLUDING ERROR)
	

	VHF RADIO (CH-15,16 & 17)
	

	NAVIGATIONAL LIGHTS & SHAPES
	

	OTHER NAVIGATIONAL EQUIPMENT
	

	2. CONFIRMATION OF SHIP’S MANEUVERABILITY
	

	3. REQUEST TO MASTER
	

	LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT
	

	FIX POSITION & REPORT PILOT IT AT PROPER TIMING
	

	MONITOR DEVIATION FROM THE ORIGINAL ROUTE
	

	CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT
	

	4. SHOW/LOWER THE FLAGS & SHAPES
	

	5. PRE-NOTICE TO MASTER
	

	REDUCTION SPEED (SEA→S/B)                          MINUTES　　　
	

	CHANGE FUEL IF NECESSARY                           MINUTES　　　
	

	THRUSTER TO USE                              　　　  MINUTES　　　
	

	STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.)   MINUTES　　　
	

	ASTERN TEST
	

	6.  SEA　SPEED　　　　　　KTS　　　　　　　　　　　RPM
	

	7.  EXCHANGE & CONFIRMATION OF
	

	“PILOT CARD” AND “PILOT INFORMATION CARD”
	

	· SELF ALCOHOL CHECK BY PILOTS
	

	   BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l.
	


MASTER




PILOT

Bound For


IWAKUNI  C-2





HAYASUI


(N)


(S)





KUDAKO


(N)


(S)











Berthing plan





Tug boats arrangement


① (                 )　　　　PS


② (                 )　　　　PS


③ (                 )　　　　PS　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　


     　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　


                       








                      　　　　　　　　　　　　　　　　　　　　　　　　　　　





                    　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　 


                   














                     ③





                           ④ 　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　　


                          





�





about 150ｍ　











During pilotage, particularly on long passage, circumstances may change which require an alteration from the Information provided.  And it should be borne in mind that pilots have their own preferred ways of working which may lead to variations.








