Revised OCT, 2015  (E)－3

LICENSED　INLAND　SEA　PILOT

For LNG ｃarrier

Bound for MEGA

PILOT　INFORMATION　CARD

Bridge　Resource　Management　Exchange　With　Pilot (s)

To　the　Master of                                 Pilot 1　:

Pilot 2　:

For　safe　navigation of your vessel you are kindly requested　to:

‐ confirm the pilotage plan on this　card,

‐ keep a sharp　lookout,　　　　　　　　　　　　　　　　　　　             Name of escort boat

‐ keep watching VHF on channel 16, and              	①

‐ put a question whenever you acknowledge a discrepancy.  	②

‐ fix position at proper interval.　　　　　　　　　　　　　　　　　　　　　 	③　　　　　　　     　　　for AKASHl T.R. only

Details of Passage Plan and relevant information

Other information

Tug boats arrangement A or B

①(            　 　)　　　 PS  　②(      　　     　 )　　　PS

③(          　 　  )  　　 PS  　④(       　     　 )　　　PS

〔Ａ〕　　　　　　　　　　　　　　　　　 　〔Ｂ〕

Safety measures for berthing

①Berthing operation should be suspended when average wind velocity exceeds 8m/sec and/or

visibility becomes less than l mile.

②In the above case, the ship should wait for recovery of weather conditions in Waiting

Anchorage (330ﾟ 3700m from Kami　Shima L.H.).

③All tugs should be made fast to the ship before beginning final approach to the jetty.

④In approaching, the ship should be made parallel at about 100m off the jetty

and then be pushed up towards the jetty.

⑤A berthing speed should be under 25cm/sec at about

50m off the jetty.

⑥Final berthing speed should be kept below 8cm/sec.

⑦When touching fenders, the ship should be completely parallel or slightly at bow-in angle

to the jetty to prevent damage to the container platform.

⑧All the lines should be taken after the ship has reached to the jetty.

⑨Whenever let go tow line, please release and down respective tow line / Heaving Line safely

onto deck of tug.( After pointing at clear deck of tug.)

Berthing Plan of Mega LNG　Terminal

Date:

Master                                      Inland Sea Pilot

During pilotage, particularly on long passage, circumstances may change which require an alteration from the　Information provided.  And it should be borne in mind that pilots have their own preferred ways of working which may lead to variations.

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT