(E)－4

LICENSED INLAND SEA PILOT

For LNG Carrier

Sail from MEGA

PILOT INFORMATION CARD

Bridge　Resource　Management　Exchange　With　Pilot (s)

To the Master of                          　　 　　　	Pilot 1:

Pilot 2:

For safe navigation of your vessel, you are kindly requested to:

－confirm the pilotage plan on this card,                      Name of escort boat

－keep a sharp lookout,                                      ①

－keep watching VHF on channel 16, and                      ②

－put a question whenever you acknowledge a discrepancy.     ③  　　　　　        　　　 for AKASHI T.R. only

－fix position at proper interval.

Detail of Passage Plan and relevant information

Other information

Tug boat arrangement

①(                　)　　　　　PS

②(                　)　　　　　PS

③(                　)　　　　　PS

④(                　)　　　　　PS

Safety measures for Unberthing

① Unberthing operation should be suspended when average wind velocity exceeds 8m/sec and/or

visibility becomes less than l mile.

② Make fast 2 tugs fore and aft on a tow line and push up to the berth together with amidships tug (no line).

③ Let go all shore lines, first head/stern lines, secondly breast lines and lastly spring lines.

④ Pull the ship by tugs bodily sideways and parallel with the berth, to about 100m off.

⑤ Then turn the ship round 180ﾟ anticlockwise, keeping the minimum distance of about 80m to the berth.

⑥ When the ships heading comes to between the breakwaters, stop turning and start proceeding with the engines and the rudder and then let go all tugs.

⑦ Increase the speed progressively so as to pass the breakwaters at about 5kts and to proceed through the

fairway at about 8kts in order to cope with cross current and/or wind.

⑧ 3 tugs will serve as escorts on her sides while the ship is transiting the fairway.

⑨　Whenever let go tow line, please release and down respective tow line / Heaving Line safely onto deck

of tug. ( After pointing at clear deck of tug. )

Plan of Mega LNG Terminal

CONTAINER PLATFORM

CAUTION!   Very frail

About 100m

Date:

Master                             　　            　Inland Sea Pilot

During pilotage,　particularly on long passage, circumstances may change which require an alteration from

the information provided and it should be borne in mind that pilotｓ has their own preferred way of working

which may lead to variations.

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT