(E)－5

LICENSED INLAND SEA PILOT

PILOT INFORMATION CARD

(WADA / SHIKAMA)

Bridge　Resource　Management　Exchange　With　Pilot (s)

To the Master of                                　　　　Pilot:

For safe navigation of your vessel, you are kindly requested to:

－confirm the pilotage plan on this card,

－keep a sharp lookout,

－keep watching VHF channel 16, and

－put a question whenever you find discrepancy.

－fix position at proper interval.

Detail of passage plan and relevant information

During pilotage, particularly on long passage, circumstances may change which require an alteration from the information provided. And it should be borne in mind that a pilot has their own preferred way of working which may lead to variations.

Tidal Height:

(        cm)

(        cm)

(        cm)

Other Information

Tug boat arrangement

(①　　　　　　   　　 　　)　　　　　PS

(②　　　　　　　   　 　　)　　　　　PS

(③　　　　　　　   　 　　)　　　　　PS

① Unberthing　operation　should　be　suspended　when　average　wind　velocity　exceeds 8m/sec　and/or　visibility　becomes　less　than　l　mile.

② Make　fast 2 tugs　fore　and　aft　on　a　tow　line　and　push　up　to　the　berth　together with　amidships　tug (no　line).

③ Let go all shore lines, first head/stern lines, secondly breast lines and lastly spring lines.

④ Pull　the　ship　by　tugs　bodily　sideways　and　parallel　with　the　berth, to　about 80m　off、

⑤ Then　turn　the　ship　round 180ﾟ anticlockwise, keeping　the　minimum　distance　of　about 80m to the　berth.

⑥ When　the　ships　heading　comes　to　between　the　breakwaters, stop　turning　and　let　go　all　tugs　and　then　start proceeding　with　the　engines　and　the　rudder.

⑦ Increase　the　speed　progressively so　as　to　pass　the　breakwaters　at　about 5kts　and　to　proceed　through　the　fairway　at　about 8kts　in　order　to　cope　with　cross　current　and/or　wind.

⑧ 3 tugs　will　serve　as　escorts　on　her　sides　while　the　ship　is　transiting　the　fairway.

Plan of Mega LNG Terminal

③ Shell bit line

1805

(w) 0827      1447    0.78k’t

(E)        0.64k’t               2119

1139

Date：

Master                               　　　      Inland Sea Pilot

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT