(M)－6

For　LNG sail from

MIZUSHIMA

LICENSED INLAND SEA PILOT

PILOT　INFORMATION　CARD

Bridge　Resource　Management　Exchange　With　Pilot (s)

To the Master of                                          　 Pilot:

Sub Pilot :

For safe navigation of your vessel, you are kindly requested to:

Name of escort boats

①　　　　　　　　　　　　　／

②　　　　　　　　　　　　　／

③　　　　　　　　　　　　for AKASHI T.R. only

－confirm the pilotage plan on this card,

－keep a sharp lookout,

－keep watching VHF channel 16, and

－put a question whenever you find discrepancy.

－fix position proper interval.

Detail of passage plan and relevant information

①Two escort boats, on one of which equipped with fire fighting apparatus, should be employed during the whole passage.

②One escort boat should be additionally employed for passing Akashi Kaikyo T.R.

Other information :

Tug boats arrangement

① (　　　　　　　　　)　　　　PS

② (　　　　　　　　　)　　　　PS

③ (　　　　　　　　　)　　　　PS

① Unberthing operation should be suspended when average wind velocity exceeds 10m/sec and/or visibility becomes less than 2000m.

② Make fast 2 tugs fore and aft on a tow line and push up to the berth together with amidships tug (no line).

③ Let go all shore lines, first head/stern lines, secondly breast lines and lastly spring lines.

④ Pull the ship by tugs bodily sideways and parallel with the berth, to about 80m off.

⑤ Then turn the ship round 180ﾟ clockwise, keeping the minimum distance of about 80m to the berth.

⑥ Whenever let go tow line, please release and down respective tow line / heaving line safely onto tug’s deck.

Date:

Master                                         　　  Inland Sea Pilot

During pilotage, particularly on long passage, circumstances may change which require an alterationfrom the information provided, and it should be borne in mind that a pilot has his own preferred wayof working　which may lead to variations.

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT