(W)－４

ＲＥＶＩＳＥＤ　Mar. 2009

LICENSED INLAND SEA PILOT

For LNG carrier

Bound For OITA

PILOT　INFORMATION　CARD

Ｂｒｉｄｇｅ　Ｒｅｓｏｕｒｃｅ　Ｍａｎａｇｅｍｅｎｔ　Ｅｘｃｈａｎｇｅ　Ｗｉｔｈ　Ｐｉｌｏｔ（ｓ）

To the Master of                          　       　　　Pilot:

H’r Pilot:

For safe navigation of your vessel, you are kindly requested to:

－confirm the pilotage plan on this card,

－keep a sharp lookout,

－keep watching VHF channel 16, and

－put a question whenever you find discrepancy.

－fix position proper interval.

Details of passage plan and relevant information.

The Safety Pledge Letter should be observed.

One escort boat should be employed from

Seki Saki pilot station to Oita LNG Terminal.

Other information

Berthing plan

Tug boats arrangement

①(                 )　　　　PS

②(                 )　　　　PS

③(                 )　　　　PS

④(                　)　　　　PS

Safety Measures for berthing

① Berthing operation should be suspended when wind velocity exceeds 10m/sec and/or visibility

becomes less than 1 mile.

② In case of the above, the ship should wait for recovery of weather conditions outside the Inland Sea.

③ All tugs should be made fast to the ship before beginning final approach to the Jetty.

④ The ship should be kept parallel against the Jetty 150m~200m off the Jetty.

⑤ The ship should be approached with parallel to the Jetty by Tugs and confirm berthing speed less than 10cm/sec at 20m~30m off the Jetty.

⑥ Final berthing speed should be kept below 5cm/sec at the best effort.

⑦ First rope should be aft spring after ship on fenders.  Fore spring is brought by line boat.

Plan of Oita LNG Terminal and mooring arrangements

Date:

During pilotage,　particularly on long passage, circumstances may change which require an alteration

From the information provided. And it should be borne in mind that a pilot has their own preferred way

of working which may lead to variations.

Master                                         Inland Sea Pilot

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT