(Ｈ)－1

For LNG carrier

SEKISAKI / HIROSHIMA

REVISED Jun. 2005

LICENSED INLAND SEA PILOT

PILOT　INFORMATION　CARD

Bridge　Resource　Management　Exchange　With　Pilot (s)

To the Master of   　　　　　　　　        　 　     Pilot:

For　safe navigation of your vessel you are kindly requested to:

‐ confirm the pilotage plan on this card,

‐ keep a sharp lookout,　　　　　　　　　　　　　　　　　　　  Name of escort boat

‐ keep watching VHF on channel 16, and

‐ put a question whenever you acknowledge a discrepancy.      　(　　　　　　　　　　　　　)

‐ fix position proper interval.

Details of Passage Plan and relevant information

Rendezvous point

with escort boat

① Ship should not proceed in Hayasui Seto or

Hiroshima Bay, if the visibility is restricted less

than 1mile.

② Ship should observe the following speed restriction

in the Safety Pledge Letter.

below 15knots---in Hayasui Seto and Iyo Nada

below 12knots---in Kudako Suido, Hiroshima Bay

Carrying dangerous cargo:            Day　　　　　　Night

International Code of Signals

1st substitute & B　　　　　　　　　　　Flashing red light

Other Information

Tug boats arrangement

① (                 )　　　　PS

② (                 )　　　　PS

③

④

Berthing plan

(　　 　kts)

)

(    kts)

(    kts)

(　 　　kts)

Safety Measures for berthing

① Berthing operation should be suspended when wind velocity exceeds 12m/sec and/or

visibility becomes less than 1 mile.

② All tugs should be made fast to the ship before beginning final approach to the jetty.

③ The ship should be made a complete and parallel stop 100m off the jetty.

④ In the process of pushing towards jetty, make a complete and parallel stop again about

50m off the jetty to take spring lines, fore and aft each, by line boats.

⑤ In view of the lightness of ships displacement, usage of excessive tug power should be avoided.

⑥ Berthing speed should be kept below 8cm/sec about 10m off the berth, and below 5cm/sec

at the final stage.

⑦ Sent stern lines after the ship got alongside or it became unnecessary to use ship’s engine.

Berthing Plan of Hiroshima LNG Terminal

DDate:

Master                                     Inland Sea Pilot

During pilotage, particularly on long passage, circumstances may change which require an alteration from the Information provided.  And it should be borne in mind that pilots have their own preferred ways of working which may lead to variations.

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT