(M)－10

LICENSED INLAND SEA PILOT

For　LNG carrier bound

for　SAKAIDE

PILOT　INFORMATION　CARD

Bridge　Resource　Management　Exchange　With　Pilot (s)

To the Master of                                            Pilot :

Sub Pilot :

For safe navigation of your vessel, you are kindly requested to:

Name of escort boats

①　　　　　　　　　　　　／

②　　　　　　　　　　　　／

③　　　　　　　　　　　　for AKASHI T.R.　only

－confirm the pilotage plan on this card,

－keep a sharp lookout,

－keep watching VHF channel 16, and

－put a question whenever you find discrepancy.

－fix position proper interval.

Detail of passage plan and relevant information

- Two escort boats,on one of which equipped with fire fighting apparatus,should be employed during the whole passage.

- One escort boat should be additionally employed for passing Akashi Kaikyo T.R.

Other information:

Tug boats arrangement

① (　　　　　　　　　　　)　　　　　PS

② (　　　　　　　　　　　)　　　　　PS

③ (　　　　　　　　　　　)　　　　　PS

④ (　　　　　　　　　　　)　　　　　PS

Safety measures for berthing

- Berthing operation should be suspended when average wind speed exceeds 10m/sec and/or visibility

becomes less than ２０００m.

- In above case, vessel should wait for recovery of weather conditions in the appropriate Waiting Anchorage

outside of the Traffic Route.

- Berthing operation should be performed during slack water.

- In approaching, vessel should be brought to about150-200m off the jetty and then be pushed up towards the jetty.

- Vessel should be made a complete and parallel stop at about 50m off the jetty.

- Final berthing speed should be kept below 5cm/sec.

Plan of Cosmo No.1 Oil Terminal and mooring arrangements

Date:

Master                                         Inland Sea Pilot

During pilotage,　particularly on long passage, circumstances may change which require an alteration

from the information provided.　And it should be borne in mind that a pilots have their own preferred way of working which may lead to variations.

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT