(Ｈ)－2－1

For LNG carrier

SEKISAKI / HIROSHIMA

REVISED Feb. 2017

LICENSED INLAND SEA PILOT

PILOT　INFORMATION　CARD

Bridge　Resource　Management　Exchange　With　Pilot (s)

To the Master of   　　　　　　　　        　 　     Pilot:

Sub Pilot:

For　safe navigation of your vessel you are kindly requested to:

‐ confirm the pilotage plan on this card,　　　　　　　　　　　　Name of escort boat

‐ keep a sharp lookout,　　　　　　　　　　　　　　　　　　　 	(　　　　　　　　　　　　　)

‐ keep watching VHF on channel 16, and                   　	(　　　　　　　　　　　　　)

‐ put a question whenever you acknowledge a discrepancy.

Rendezvous point

with escort boat&tug.

(Escort tug with fire

Fighting equip.)

Details of Passage Plan and relevant information

(N)

(S)

KUDAKO SUIDOU

① Ship should not proceed in Hayasui Seto or

HAYASUI SETO

Hiroshima Bay, if the visibility is restricted less

(N)

(S)

than 1mile.

② Ship shall be scheduled the following speed.

below   knots---in Hayasui Seto and Iyo Nada

below   knots---in Kudako Suido, Hiroshima Bay

The Safety Pledge Letter should be observed.

Other Information

Berthing plan

Tug boats arrangement

① (                 )　　　　PS

② (                 )　　　　PS

③ (                 )　　　　PS

④ (                 )　　　　PS

③

④

Safety Measures for berthing

① Berthing operation should be suspended when wind velocity exceeds 8m/sec and/or

visibility becomes less than 1 mile.

② All tugs should be made fast to the ship before beginning final approach to the jetty.

③ The ship should be made a complete and parallel stop 200m off the jetty.

④ In the process of pushing towards jetty, make a complete and parallel stop again about

50m off the jetty to take spring lines, fore and aft each, by line boats or messenger rope.

⑤ Berthing speed should be kept below 5cm/sec at the final stage.

⑥ Sent stern lines after the ship got alongside and it became unnecessary to use ship’s engine.

Berthing Plan of Hiroshima LNG Terminal

Date:

Master                                     Inland Sea Pilot

During pilotage, particularly on long passage, circumstances may change which require an alteration from the Information provided.  And it should be borne in mind that pilots have their own preferred ways of working which may lead to variations.

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT