(Ｈ)－２－２

For LNG carrier

Sail From HIROSHIMA

REVISED Feb.2017

LICENSED INLAND SEA PILOT

PILOT　INFORMATION　CARD

Bridge　Resource　Management　Exchange　With　Pilot (s)

To the Master of   　　　　　　　　        　 　     Pilot:

Sub Pilot:

For　safe navigation of your vessel you are kindly requested to:

‐ confirm the pilotage plan on this card,　　　　　　　　　　　　Name of escort boat

‐ keep a sharp lookout,　　　　　　　　　　　　　　　　　　　 	(　　　　　　　　　　　　　)

‐ keep watching VHF on channel 16, and                   　	(　　　　　　　　　　　　　)

‐ put a question whenever you acknowledge a discrepancy.

Escort boat&tug.

(Escort tug with fire

Fighting equip.)

Details of Passage Plan and relevant information

① Ship should not proceed in Hayasui Seto or

Hiroshima Bay, if the visibility is restricted less

than 1mile.

② Ship shall be scheduled the following speed.

below   knots---in Hayasui Seto and Iyo Nada

below   knots---in Kudako Suido, Hiroshima Bay

The Safety Pledge Letter should be observed.

Other Information

Tug boats arrangement

① (                 )　　　　PS

② (                 )　　　　PS

③ (                 )　　　　PS

④ (                 )　　　　PS

③

④

UNBERTHING PLAN

※4tug boats should be arranged when average wind velocity is between 8m/s and 10m/s.

※3tug boats should be arranged when average wind velocity is 8m/s . And/or less than 8m/s.

Safety Measures for berthing

① Unberthing operation should be suspended when average wind velocity exceeds 10m/sec and/or visibility becomes less than l mile.

② Make fast 2 tugs fore and aft on a tow line and push up to the berth together with amidships tug (no line).

③ Let go all shore lines, first head/stern lines, secondly breast lines and lastly spring lines.

④ Pull the ship by tugs bodily sideways and parallel with the berth, to about 150m off and the ship should be towed to about 0.5L east off-shore from the jetty.

⑤ Then turn the ship round 180ﾟ anticlockwise, keeping the safety distance to the berth.

⑥ When the ships heading comes to outside of the port, Increase the speed stop turning and then let go all tugs.

Plan of Hiroshima LNG Terminal

Date:

Master                                     Inland Sea Pilot

During pilotage, particularly on long passage, circumstances may change which require an alteration from the Information provided.  And it should be borne in mind that pilots have their own preferred ways of working which may lead to variations.

CHECK  LIST  UNDER  PILOTAGE

DATE           :

SHIP’S  NAME  :

Voy.No

| - CONFIRMATION OF SHIP’S CONDITION & EQUIPMENT |  |
| --- | --- |
| MAIN ENGINE |  |
| SHIP’S DRAFT F： (M： ) A： |  |
| WHISTLE (IF AIR SUPPLY & LOCATION) |  |
| THRUSTER IF INSTALL |  |
| STEERING GEARS UNDER OPERATION (TWO UNITS) |  |
| GYRO COMPASS (INCLUDING ERROR) |  |
| VHF RADIO (CH-15,16 & 17) |  |
| NAVIGATIONAL LIGHTS & SHAPES |  |
| OTHER NAVIGATIONAL EQUIPMENT |  |
| - CONFIRMATION OF SHIP’S MANEUVERABILITY |  |
| - REQUEST TO MASTER |  |
| LOOKOUT & MONITOR OTHER SHIP’S MOVEMENT |  |
| FIX POSITION & REPORT PILOT IT AT PROPER TIMING |  |
| MONITOR DEVIATION FROM THE ORIGINAL ROUTE |  |
| CONFIRMATION & MONITOR PILOT’S INTENTION/ORDER IF ANY DOUBT |  |
| - SHOW/LOWER THE FLAGS & SHAPES |  |
| - PRE-NOTICE TO MASTER |  |
| REDUCTION SPEED (SEA→S/B) MINUTES |  |
| CHANGE FUEL IF NECESSARY MINUTES |  |
| THRUSTER TO USE MINUTES |  |
| STATION CREW (TUG LINE , ANCHOR , BERTHING ETC.) MINUTES |  |
| ASTERN TEST |  |
| 6. SEA SPEED KTS RPM |  |
| 7. EXCHANGE & CONFIRMATION OF |  |
| “PILOT CARD” AND “PILOT INFORMATION CARD” |  |
| - SELF ALCOHOL CHECK BY PILOTS |  |
| BREATH ALCOHOL CONCENTRATION MEASURED IS LESS THAN 0.15mg/l. |  |

MASTER					PILOT